Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             116

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

No person shall within any Highway Commercial (C2) Zone use any land or erect, alter or use any buildings or structure except in accordance with the following provisions.

  - PERMITTED USES

    - Residential Uses

a)	a single detached dwelling house shall only be permitted where such dwelling is accessory to a permitted non-residential use and is occupied by the owner, caretaker, watchman or other similar persons as are employed on the lot on which such dwelling house is located;

b)	a dwelling unit in a portion of a non-residential building, except where the building is used for a motor vehicle station, a motor vehicle repair garage, a motor vehicle dealership, or a motor vehicle body shop;

c)	a home business.

    - Non-Residential

a)	auction barn or flea market

b)	builder’s supply outlet, hardware, paint and wallpaper store, home furnishings

store, appliance store

c)	business, professional or administrative office

d)	convenience store

e)	eating establishment, including a drive thru or take-out eating establishment and tea

room

f)	equipment sales, rental and repair establishment

g)	farm implement and equipment sales and service establishment

h)	food supermarket

i)	funeral home

j)	garden and nursery sales and supply establishment; greenhouse, commercial

k)	Liquor Control Board outlet, Brewers’ Retail outlet

l)	motel or hotel

m)	motor vehicle dealership, motor vehicle gasoline bar, motor vehicle repair garage,

motor vehicle service station, or a motor vehicle wash, automatic

n)	outside display and storage of goods and materials where such use is accessory and incidental to a permitted non-residential use specified hereunder;

o)	place of entertainment or recreation, including a commercial club

p)	propane refill station

q)	public transportation depot

r)	public use or municipal utility in accordance with the provisions of 4.25 set forth in this By-law.

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             117

s)	recreational vehicle, snowmobile, marine or travel trailer sales and service

establishment

t)	taxi or bus depot

u)	veterinarian clinic

v)	workshop, custom workshop

w)	parking lot

x)	any use permitted in the Community Facility (CF) Zone

y)	Bank or financial institution

z)	Dry cleaning establishment

aa)	Retail commercial establishment

    - Accessory Uses

Uses, buildings and structures accessory to any of the foregoing uses permitted under 16.1.1 and 16.1.2 hereof and in accordance with the provisions of 4.1 as set forth in this By-law.

  - REGULATIONS FOR RESIDENTIAL USES

All regulations for permitted residential uses as set out in Section 9.2 of this By-law shall apply within the Highway Commercial (C2) Zone, subject to the spatial separation requirements stated hereunder in 16.3 of this By-law.

  - SPATIAL SEPARATION REQUIREMENTS

Where a dwelling house is erected or any building or structure is altered or used for the purposes of a dwelling house, as permitted in accordance with the provisions of 16.1.1 hereof, such dwelling house shall not be erected nor shall any building be altered or used for such purposes within 3 m (10 ft.) of a permitted non-residential use, however, in no case shall a dwelling house be erected, altered or used within 7.5 m (25 ft.) of a motor vehicle service station, a motor vehicle repair garage or a motor vehicle dealership.

  - REGULATIONS FOR NON-RESIDENTIAL USES EXCLUSIVE OF RETAIL GASOLINE ESTABLISHMENTS AND MOTELS

    - Minimum Lot Area Requirement

a)	public water and sewage disposal			696 m² (7,500 sq. ft.)

b)	public water supply and private sewage disposal	1110 m² (12,000 sq. ft.) plus

185 m² (2,000 sq. ft.) where a

dwelling unit located in a

portion of a Non-Residential

building.

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             118

c)	private water supply and sewage disposal		2040 m² (22,000 sq. ft.) plus

370 m² (4,000 sq. ft.) where a

dwelling unit located in a

portion of a Non-Residential

building.

16.4.2	Minimum Lot Frontage Requirement			38 m (125 ft.)

16.4.3	Minimum Yard Requirements

a)	front yard depth					7.5 m (25 ft.)

b)	exterior side yard width				7.5 m (25 ft.)

c)	interior side yard width

i.	Abutting a Residential Zone		9 m (30 ft.)

ii.	Abutting any other Zone			6 m (20 ft.)

d)	rear yard depth					7.5 m (25 ft.)

16.4.4	Maximum Lot Coverage of All Buildings			25 percent

    - Minimum Setback from Street Centreline

a)	Provincial Highway				40.5 m (133 ft.)

b)	Municipal Road					17.5 m (57.5 ft.)

16.4.6	Minimum Landscaped Open Space Requirement		10 percent

16.4.7	Maximum Height of Buildings				11 m (36 ft.)

    - All buildings are to be faced with brick or brick-like block work.

  - REGULATIONS FOR RETAIL GASOLINE ESTABLISHMENTS

As set out in 15.6 of this By-law.

  - REGULATIONS FOR HOTELS

As set out in 15.7 of this By-law.

  - REGULATIONS FOR MOTELS

    - Minimum Lot Area Requirement

a)	public water supply and sewage disposal		950 m² (10,255 sq. ft.)

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             119

b)	public water supply and private sewage disposal	2780 m² (30,000 sq. ft.) plus

185 m² (2,000 sq. ft.) where a

dwelling unit is located in a

portion of a Non-Residential

building

c)	private water supply and sewage disposal		4000 m² (43,000 sq. ft.) plus

370 m² (4,000 sq. ft.) where a

dwelling unit is located in a

portion of a Non-Residential

building

    - Minimum Lot Frontage Requirements

a)	public water supply and sewage disposal		38 m (125 ft.)

b)	public water supply and private sewage disposal	45 m (150 ft.)

c)	private water supply and sewage disposal		53 m (175 ft.)

    - Minimum Yard Requirements

a)	front yard depth					12 m (40 ft.)

b)	exterior side yard width				12 m (40 ft.)

c)	interior side yard width

i.	Abutting a Residential Zone		9 m (30 ft.)

ii.	Abutting any other Zone			6 m (20 ft.)

d)	rear yard depth					7.5 m (25 ft.)

16.7.4	Maximum Lot Coverage of All Buildings			25 percent

16.7.5	Minimum Setback from Street Centreline

a)	Provincial Highway				32 m (105 ft.)

b)	Municipal Road					25 m (82 ft.)

16.7.6	Minimum Landscaped Open Space Requirement		10 percent

16.7.7	Maximum Height of Buildings				11 m (36 ft.)

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             120

    - Distance Between Buildings

Where two or more buildings are erected on the same lot, the minimum distance between buildings shall not be less than the minimum distance which would be required if each building was on a separate lot and subject to the minimum yard requirements stated in 16.7.3 of this By-law.

    - Exterior Wall

Where the exterior wall of a guest room contains a habitable room window such wall shall be located not closer than 7.5 m (25 ft.) to any exterior side or rear lot line

    - All buildings are to be faced with brick or a brick-like block work.

  - BUFFER STRIP REQUIREMENTS

Where the interior side or rear lot line of a lot within a Highway Commercial (C2) Zone abuts a Residential Zone, an area adjoining such abutting lot line shall be used for no other purpose than for a buffer strip in accordance with the requirements of 4.5 set forth in this By-law.

  - GENERAL ZONE PROVISIONS

All provisions of Section 4, General Zone Provisions, of this By-law, where applicable to the use of any land, building or structure permitted within the Highway Commercial (C2) Zone shall apply.

  - LOADING AND PARKING PROVISIONS

All provisions of Section 5, Loading and Parking Provisions, of this By-law, where applicable to the use of any land, building or structure permitted within the Highway Commercial (C2) Zone shall apply.

  - EXCEPTION ZONE PROVISIONS – HIGHWAY COMMERCIAL (C2) ZONE

    - Highway Commercial Exception No. 1 (C2-1) Zone, Part of Lot 11, Concession 13

Notwithstanding any provisions of this By-law to the contrary, within the Highway Commercial Exception No. 1 (C2-1) Zone, the following provisions shall apply:

i.	The only permitted uses shall be:

1.	Marine and/or snowmobile dealer

2.	Take-out eating establishment

3.	One single detached dwelling

ii.	A commercial boat launching facility is a prohibited use.

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             121

iii.	Marina facilities shall be limited to the existing wall and pier.

iv.	No building or structure shall be located or used within 8 metres (26.2 feet) of the high water mark of Moira Lake.

v.	No building or structure shall be located or used with 8 metres (26.2 feet) of the old County Road.

- No buildings or structures shall be located or used except in compliance with Schedule ‘A’ to By-law No. 2164-87 of the former Township of Huntingdon.

All other requirements of this By-law shall apply to land zoned C2-1.

16.11.2	Highway Commercial Exception No. 2 (C2-2) Zone, Part of Lot 7, Concession 8

Notwithstanding any provisions of this By-law to the contrary, within the Highway Commercial Exception No. 2 (C2-2) Zone, the following provisions shall apply:

i.	The only permitted uses shall be:

1.	One single detached dwelling

2.	Retail sale of light construction equipment and machinery

All other requirements of this By-law shall apply to land zoned C2-2.

16.11.3	Highway Commercial Exception No. 3 (C2-3) Zone, Part of Lot 12, Concession 14

Notwithstanding any provisions of this By-law to the contrary, within the Highway Commercial Exception No. 3 (C2-3) Zone, the following provisions shall apply:

i.	Minimum lot frontage			28 m (91.8 ft.)

All other requirements of this By-law shall apply to land zoned C2-3.

    - Highway Commercial Exception No. 4 (C2-4) Zone, Part of Lots 19, 20 and 21, Plan 13 (2003-12)

Notwithstanding the permitted uses and regulations for non-residential uses of the Highway

Commercial (C2) Zone to the contrary, within the Highway Commercial Exception No. 4 (C2-4)

Zone the following shall apply:

- Permitted Uses

  - business, professional or administrative office

  - convenience store

  - eating establishment, including a drive thru or take-out eating establishment and tea room

  - food supermarket

  - funeral home

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE

Municipality of Centre Hastings

Zoning By-law No. 2002-10

May 13, 2002                                                                                                                                             122

  - Liquor Control Board outlet, Brewer’s Retail outlet

  - motel or hotel

  - public use or municipal utility in accordance with the provisions of Section 4.25 set forth in this By-law

  - veterinarian clinic

  - parking lot

  - any use permitted in the Community Facility (CF) Zone

  - bank or financial institution

  - retail commercial establishment

Regulations

i.	Minimum lot frontage			33.30 m (109.26 ft.)

ii.	Minimum yards for all buildings

a)	Front yard depth			6.3 (20.5 ft.)

b)	Interior yard width		2.3 m (7.6 ft.)

c)	Rear yard depth			6.9 m (22.9 ft.)

All other requirements of this By-law shall apply to land zoned C2-4.

    - Highway Commercial Exception No. 5 (C2-5) Zone, Lots 9, 10, 11 and 12, Plan 105 (2004-23)

Notwithstanding the permitted uses and regulations for non-residential uses of the Highway Commercial (C2) Zone to the contrary, within the Highway Commercial Exception No. 5 (C2-5) Zone the following shall apply:

i.	Minimum lot frontage		20.1 m (66 ft.)

ii.	Minimum exterior side yard	4.5 m (15 ft.)

iii.	Minimum interior side yard	0.0 m (0 ft.)

iv.	Maximum lot coverage		32.0%

- That the area between the Quonset hut and the car wash building, being

some 13.7 feet wide by 165 feet long all in accordance with Schedule “B” attached hereto and by this reference forming part of this By-law, shall be kept free from any encumbrances to allow access by the Municipality to an existing storm water drainage system buried therein as may be required from time to time.

All other requirements of this By-law shall apply to land zoned C2-5.

SECTION 16 HIGHWAY COMMERCIAL (C2) ZONE